| Standard: Marbles will be crystallized with the right method to ensure clean and shining marble at all times. | 标准： 使饭店的理石得到合理的保养。保持清洁光亮。 |
| --- | --- |
| Procedures: - Prepare all necessary accessories. - Single Scrub Machine b) Wet/dry vacuum cleaner c) Needle Base d) Red Mat e) Measure cup f) Bucket g) Notice sign h) Polishing machine - Place notice signs around cleaning area. - Wash the cleaning area thoroughly with single scrub machine and purpose cleaner. - Compound the crystal power 1:1 in measure glass with water. Apply on the marble evenly. NOTE: The area should be around 3-5m2 at a time. - Fix needle base and red mat on the single scrub machine. Turn on the machine, rubbing the areas applied the compound until the marble become shinning. - After crystallization, wash the area again with single scrub machine and absorb the dirty water with wet vacuum cleaner. - Polish marble with polishing machine. - Clean all accessories and take them back to the storeroom. | 程序： - 准备清洁所需的全部工具。 - 单擦机 b. 吸水机 c. 针盘 d. 红垫 e. 量杯 f. 水桶 g 告示牌 h 抛光机 - 在清洁区域周围设置告示牌。 - 先将清洁区域彻底洗净。 - 用量杯以1:1配制结晶粉。均匀地涂在理石上。注意每次面积不要太大(3-5M2) - 使用单擦机。安装针盘、红垫。在涂好结晶粉的区域研磨，直至地面光亮。 - 结晶完毕后，用单擦机将地面洗净，并用吸水机吸干污水。 - 用抛光机抛光地面。 - 清洗全部的工具，存放在库房。 |