| Standard: All carpets should be maintained clean at all times and shampoo regularly. | 标准： 饭店内所有区域的地毯定期清洗，保持高标准的清洁水平。 |
| --- | --- |
| Procedures: - Vacuum thoroughly the areas which need shampooing. Remove all spots in advance. - Prepare all accessories: Single Scrub Machine, foam tank, brushes, dry foam cleaner, and upright vacuum. - Dilute concentrate dry foam cleaner according to situation (normally1: 20). - Move the machine to the inside of cleaning area, fix brush and switch it on. - Adjust the operation handle. - Turn on the foam tank to the highest power grade. When foam starts coming out from the brush, start the machine and turn the power to the middle power grade for saving chemicals. - Shampoo carpet from left to right. Note: the nearest two equal lines must be overlapped, avoid somewhere omitted. - When finished shampooing, cover the carpet with discarded linen to avoid being dirty when it is wet. (Use air dryer if necessary) - When the carpet is thoroughly dry, vacuum the carpet with an upright vacuum. - Take back all accessories to the storeroom. - If any problem occurs when operating, report to PA Shiftleader as soon as possible. | 程序： - 先将所要清洗的区域彻底吸尘。去除局部污渍。 - 准备所需的全部工具：单擦机、泡箱、地毯刷、高泡地毯香波、直立式吸尘器。 - 用温水稀释地毯水(1:20)倒入打泡箱内。 - 将单擦机推至清洗区域的最里端，安装好地毯刷，插好电源。 - 调节操作手柄。 - 打开泡箱最大档。当泡沫从刷子下面出来后启动机器。将泡箱开关调至中档。减少清洁剂的使用并提高工作效率。 - 一行一行从左至右清洗。注意第二行要与第一行重叠，不要遗漏。 - 待地毯洗过后，用报废床单将清洗区域铺好，防止踩脏(必要时使用吹风机) - 地毯干透后用直立式吸尘器彻底吸尘。 - 收好全部工具放回库房。 - 及时向PA领班报告工作中出现的问题。 |