| Standard: Marble should be waxed regularly and maintained clean and shining at all times. | 标准： 饭店的理石得到合理的保养。保持清洁、光亮的状态。 |
| --- | --- |
| Procedures: - Prepare all necessary accessories: Single scrub machine Wet vacuum cleaner Seal mop Bucket water extraction cart discarded towel Needle Base wax stripper floor sealer wax floor finish wax black & red mat - Place signs as barricades around sealing area. - Remove old wax of sealing: dilute the wax stripper at 1:8. Use mop to spread the wax stripper on the surface of marble evenly, and let it take effect for 5~8 minutes. Remove the old wax with single scrub machine and black mat until it is gone. Wash floor with clean water. Reseal the marble when it is thoroughly dry. - Place the rubbish bag on the popeimer, and pour the wax in. Immerse the mop into the floor sealer and absorb until no more dripping. - From the inside of the sealing area, make a frame. - Flatten the mop, mopping lines by lines inside the frame. Move backwards towards the entrance overlap the strokes. Avoid having puddles of sealer on the floor. - Do second seal when the first seal is thoroughly dry. Totally, 2 coats of floor sealer, 3 coats of floor finish wax should be done. - When finished, put away the machines and tools in the storeroom. | 程序： - 准备落蜡所需的工具。 单擦机 吸水机 蜡拖 水桶 榨水车 报废毛巾 针盘 起蜡水 底蜡 面蜡 黑垫、红垫各一块 - 在打蜡区域周围放置告示牌。 - 起蜡：以1:8稀释起蜡水。用拖把浸湿均匀的涂在蜡上。作用约5~8分钟。用单擦机配黑垫摩擦蜡面，直至蜡面清除。然后用清水清洗。待地面完全干透后再落蜡。 - 将垃圾袋套在榨水车上。将蜡水倒入其中。用清水将蜡拖头浸湿再榨干。放在蜡水中，浸满蜡水后榨干(不滴为止) - 在落蜡区域的最里端。先在整个区域上打一个框。 - 平展蜡拖，在框中来回摆动。倒退行走。每一次都要与上一次重叠。注意：速度要均匀。避免积堆。 - 待第一遍完全干透后再落第二遍。共计2遍底蜡3遍面蜡。 - 全部结束后。收拾全部工具清洁干净放回库房。 |